Work Instruction: Terminating Grounds and Shields for VFD Cable on a 900 kW Motor (Subchapter M Work Boat)

1. Scope

This instruction applies to the installation of shielded Variable Frequency Drive (VFD) power cables connecting Danfoss drives to a 900 kW Marelli motor aboard a Subchapter M towboat (non-classed). It focuses on proper termination of cable shields and ground conductors to ensure electromagnetic compatibility (EMC), minimize common-mode currents, and comply with applicable standards.

2. Safety and Compliance

- De-energize and lock out all equipment before beginning work.

- Follow Danfoss and Marelli manufacturer installation manuals.

- Comply with applicable standards:

  - NEC (NFPA 70) Article 250 (Grounding and Bonding)

  - NFPA 79 (Electrical Standard for Industrial Machinery)

  - IEEE 45 (Recommended Practice for Electrical Installations on Shipboard)

  - IEC 60092-352/353/354 (Shipboard Power Cable Standards)

  - ABS and DNV marine electrical guidelines (for best practices)

3. Materials and Tools

- Shielded VFD cable (foil + braid or copper tape shield)

- EMC-rated cable glands or shield clamps (360° contact)

- Shield termination kits (e.g., Southwire SPEC 85451)

- Grounding lugs, braid straps, constant-force springs

- Cable stripping and crimping tools

- Torque wrench, electrical tape, heat shrink tubing

4. Termination Procedure

4.1 General Cable Routing

- Route VFD cable away from sensitive electronics.

- Avoid sharp bends; maintain minimum bend radius.

- Use metallic conduit or tray as needed, bonded at both ends.

- Avoid splices; maintain shield continuity.

4.2 VFD End Termination

- Strip jacket to expose shield and conductors.

- Fold braid back over jacket; trim foil and drain wire.

- Use EMC gland or shield clamp to achieve 360° shield contact.

- Terminate PE ground conductor to VFD ground terminal.

- Connect phase conductors (U/V/W) per Danfoss manual.

4.3 Motor End Termination

- Prepare motor junction box for shield bonding (remove paint or use cutting locknuts).

- Strip jacket and prepare shield as above.

- Use EMC gland or clamp to bond shield to motor enclosure.

- Terminate PE ground conductor to motor ground lug.

- Connect phase conductors to motor terminals (U/V/W).

4.4 Intermediate Disconnects or Junctions (if applicable)

- Maintain shield continuity using:

  - Isolated shield terminal blocks, or

  - Dual EMC glands bonded to enclosure, or

  - Short pigtails (only if necessary), combining braid + drain, kept as short as possible.

5. Best Practices

| Practice | Recommendation |
| --- | --- |
| Shield Termination | Terminate at both VFD and motor ends using 360° contact methods. |
| Ground Conductors | Terminate all PE conductors at both ends. For 3G cables, combine grounds into a single lug if needed. |
| Shield Continuity | Maintain uninterrupted shield path through all enclosures. |
| Pigtails | Avoid if possible. If used, keep short and include braid + drain. |
| Bonding Surfaces | Ensure metal-to-metal contact; remove paint or use cutting locknuts. |
| Documentation | Label all terminations and update vessel electrical drawings. |
| Testing | Verify continuity and insulation resistance. Check for EMI during commissioning. |

References

- Danfoss VLT® 2800 Series Installation Manual – Grounding of Shielded/Armoured Control Cables

- Marelli Motori Three-Phase Motors Installation Instructions

- Southwire Application Note: “Begin with the End in Mind – Proper VFD Cable Termination”

- Southwire SPEC 85451 – VFD Shield Termination Kit Specification

- LUTZE Driveflex® Shield Termination Guide

- ABB Drives Reference Manual: “Grounding and Cabling of Drive Systems” (3AFY61201998 Rev D)

- NFPA 79 – Electrical Standard for Industrial Machinery

- NEC (NFPA 70) – Article 250: Grounding and Bonding

- IEEE Std 45 – Recommended Practice for Electrical Installations on Shipboard

- IEC 60092 Series – Electrical Installations in Ships

- DNV and ABS Marine Electrical Installation Guidelines